site-packagesv

- gnuradio

  - blks:

| __init__.py | This file is required to make Python treat the directories as packages |
| --- | --- |

  - blksimpl:

| am_demod.py | AM demodulation block |
| --- | --- |
| dbpsk.py | Differential BPSK modulation and demodulation |
| dqpsk.py | Differential QPSK modulation and demodulation |
| filterbank.py | Include both synthesis and analysis |
| fm_demod.py | Generalized FM demodulation block |
| fm_emph.py | FM deemphasis and preemphasis IIR filter |
| gmsk.py | GMSK modulator and demodulator |
| __init__.py | nothing |
| nbfm_rx.py | Narrow band FM receiver |
| nbfm_tx.py | Narrow band FM transmitter |
| pkt.py | mod/demod with packets as i/o (sending packets and demodulating/deframing packets) |
| psk.py | define different kinds of PSK (BPSK, QPSK, 8PSK) |
| rational_resampler.py | Rational resampling polyphase FIR filter |
| standard_squelch.py | Implement the squelch function |
| wfm_rev.py | Demodulating a broadcast FM signal |
| wfm_rcv_pll.py | Demodulating a broadcast FM signal with stereo |
| wfm_tx.py | Wide band FM Transmitter |

  - gr

| basic_flow_graph.py | Constructs the basic graph and provides basic operations on the graph (base class of “flow_graph”) |
| --- | --- |
| exceptions.py | Exception handling (not finished yet) |
| flow_graph.py | Define the type of flow graph and play an key role in scheduling the signal control |
| gnuradio_swig_python.py | Automatically created by SWIG |
| gr_threading.py | Choose load gr_threading_23.py or gr_threading_24.py |
| gr_threading_23.py | Threading module for version 2.3 |
| gr_threading_24.py | Threading module for version 2.4 |
| hier_block.py | Construct hierarchical blocks for graph (many non-implemented modules) |
| __init__.py |  |
| prefs.py |  |
| scheduler.py | Schedule the threads |

  - gru:

| __init__.py |  |
| --- | --- |

  - gruimpl:

| crc.py | CRC generator |
| --- | --- |
| freqz.py | Compute frequency response of a digital filter given the numerator (b) and denominator (a) |
| gnuplot_freqz.py | Plot the frequency response by using gnuplot |
| hexint.py | Convert unsigned masks into signed ints |
| __init__.py |  |
| listmisc.py | Return a copy of input ‘x’ that is reverse order |
| lmx2306.py | Control National IMX2306 (specific hardware) based frequency synthesizer |
| mathmisc.py | Some math functions (Gcd, Lcm, Log2) |
| os_read_exactly.py | Replace os.read that blocks until it reads exactly nbytes. |
| sdr__1000.py | Control the DDS(direct digital synthesizer) on the SDR-1000 (specific hardware) |
| seq_with_cursor.py |  |
| socket_stuff.py | Set up socket for TCP/UDP connections |

  - pager:

| aypabtu.py | Mostly same as usrp_flex.py |
| --- | --- |
| flex_demod.py | FLEX pager protocol demodulation block |
| __init__.py |  |
| pager_swig.py | Automatically created by SWIG |
| usrp_flex.py | This is an example to demonstrates receiving and demodulating the FLEX pager protocol |

  - vocoder:

| gsm_full_rate.py | Automatically created by SWIG |
| --- | --- |
| __init__.py | Not implemented yet |

  - wxgui:

| ftsink.py | FFT sink test application |
| --- | --- |
| form.py | wxgui form demo (Editbox, Int/Float check) |
| __init__.py |  |
| plot.py | Demonstrate the drawing function |
| powermate.py | Handler for Griffin PowerMate, Contour ShuttlePro & ShuttleXpress USB knobs |
| ra_fftsink.py | FFT sink test application |
| ra_stripchartsink.py |  |
| ra_waterfallsink.py |  |
| scopesink.py | O’Scope Test Application |
| slider.py | Demonstrate how to control slider |
| stdgui.py | A simple wx gui for GNU Radio application |
| waterfallsink.py | Waterfall sink test application |

  - atsc.py: Automatically created by SWIG

  - audio.py: Generic audio or soundcard interface

  - audio_alsa.py: Automatically created by SWIG

  - audio_oss.py: Automatically created by SWIG

db_ = daughterboard

  - db_base.py: Abstract base class for all daughterboards. This defines the required operations and interfaces for all d'boards.

  - db_basic.py: Handler for basic Tx daughterboards, Basic Rx daughterboards, Low Freq Rx daughterboards, Low Freq Tx daughterboards.

  - db_dbs_rx.py: Control DBS receiver based USRP daughterboard.

  - db_flexrf: Functions for all flexrf boards.

  - db_flexrf_mimo.py: Mimo classes for all flexrf boards.

  - db_instantiator.py: Instantiate for accessing daughterboard

  - db_tv_rx.py: Control Microtune 4937 based USRP daughterboard

  - ecc.py: Automatically created by SWIG

  - eng_notation.py: Change engineering notation (E.g., 5e-9 -> 5n)

  - eng_option.py: Add support for engineering notation

  - flexrf_debug_gui.py: Debug GUI for ‘Flexrf’

  - gr_unittest.py: Add support for unittest

  - __init__.py:

  - local_calibrator.py: NO LONGER USED

  - modulation_utils.py: Miscellaneous utilities for managing modulations and

demodulations

  - optfir.py: Routines for designing optimal FIR filters

  - packet_utils.py: Utilities for packet handling

  - ra.py: Automatically created by SWIG

  - trellis.py: Automatically created by SWIG

  - tx_debug_gui.py: Debug tool for ‘Tx’

  - usrp.py: Basic configuration for USRP

  - usrpl.py: Automatically created by SWIG

  - usrp_multi.py: Multiple-source handling for USRP

  - video_sdl.py: Automatically created by SWIG

  - window.py: Routines for designing window functions

- __init__.py:

- usrp_dbid.py: Defines USRP daughterboard ID

- usrp_fpga_regs.py: Copy everything that starts with FR_ or bmFR_ from the

usrp_prims name space into our name space

- usrp_prims.py: Automatically created by SWIG